HACETTEPE UNIVERSITY

DEPARTMENT OF ELECTRICAL AND ELECTRONICS ENGINEERING

ELE 432: Advanced Digital Design

Homework 2

Hakan Töre

2210357024

- I have spent 5 hours on this homework. Although most of the time was spent on figuring out why the ALUControl values were off in the questa transcript. The controller.tv test vector file was wrong and did not match the architectrue given to us by this course so we had to update the test vectors.

/*

Hakan Töre 2210357024

ELE432 Homework 2 : HW2_Multi-Cycle_RISC-V_Processor

controller.sv

*/

module controller(

input logic 		 clk,

input logic 		 reset,

input logic  [6:0] op,

input logic  [2:0] funct3,

input logic 		 funct7b5,

input logic 		 zero,

output logic [1:0] immsrc,

output logic [1:0] alusrca, alusrcb,

output logic [1:0] resultsrc,

output logic 		 adrsrc,

output logic [2:0] alucontrol,

output logic 		 irwrite, pcwrite,

output logic 		 regwrite, memwrite

);

logic [1:0] aluop;

logic       branch;

logic       pcupdate;

assign pcwrite = pcupdate | (branch & zero);

instrdec id (

.op(op),

.ImmSrc(immsrc)

);

aludec ad (

.opb5(op[5]),

.funct3(funct3),

.funct7b5(funct7b5),

.ALUOp(aluop),

.ALUControl(alucontrol)

);

mainfsm fsm (

.clk(clk),

.reset(reset),

.op(op),

.Branch(branch),

.PCUpdate(pcupdate),

.RegWrite(regwrite),

.MemWrite(memwrite),

.IRWrite(irwrite),

.ResultSrc(resultsrc),

.ALUSrcA(alusrca),

.ALUSrcB(alusrcb),

.AdrSrc(adrsrc),

.ALUOp(aluop)

);

endmodule

/*

Hakan Töre 2210357024

ELE432 Homework 2 : HW2_Multi-Cycle_RISC-V_Processor

mainfsm.sv

*/

module mainfsm (

input  logic       clk,

input  logic       reset,

input  logic [6:0] op,

output logic       Branch,

output logic       PCUpdate,

output logic       RegWrite,

output logic       MemWrite,

output logic       IRWrite,

output logic [1:0] ResultSrc,

output logic [1:0] ALUSrcA,

output logic [1:0] ALUSrcB,

output logic       AdrSrc,

output logic [1:0] ALUOp

);

// State Encoding

typedef enum logic [3:0] {

S0_FETCH    = 4'b0000,

S1_DECODE   = 4'b0001,

S2_MEMADR   = 4'b0010,

S3_MEMREAD  = 4'b0011,

S4_MEMWB    = 4'b0100,

S5_MEMWRITE = 4'b0101,

S6_EXECUTER = 4'b0110,

S7_ALUWB    = 4'b0111,

S8_EXECUTEI = 4'b1000,

S9_JAL      = 4'b1001,

S10_BEQ     = 4'b1010

} statetype;

statetype state, nextstate;

// State Register

always_ff @(posedge clk or posedge reset) begin

if (reset) state <= S0_FETCH;

else       state <= nextstate;

end

// Next State Logic

always_comb begin

case (state)

S0_FETCH:    nextstate = S1_DECODE;

S1_DECODE: begin

case (op)

7'b0000011: nextstate = S2_MEMADR;   // lw

7'b0100011: nextstate = S2_MEMADR;   // sw

7'b0110011: nextstate = S6_EXECUTER; // R-type

7'b0010011: nextstate = S8_EXECUTEI; // I-type ALU

7'b1101111: nextstate = S9_JAL;      // jal

7'b1100011: nextstate = S10_BEQ;     // beq

default:    nextstate = S0_FETCH;

endcase

end

S2_MEMADR: begin

case (op)

7'b0000011: nextstate = S3_MEMREAD;  // lw

7'b0100011: nextstate = S5_MEMWRITE; // sw

default:    nextstate = S0_FETCH;

endcase

end

S3_MEMREAD:  nextstate = S4_MEMWB;

S4_MEMWB:	 nextstate = S0_FETCH;

S5_MEMWRITE: nextstate = S0_FETCH;

S6_EXECUTER: nextstate = S7_ALUWB;

S7_ALUWB:    nextstate = S0_FETCH;

S8_EXECUTEI: nextstate = S7_ALUWB;

S9_JAL:    	 nextstate = S7_ALUWB;

S10_BEQ:   	 nextstate = S0_FETCH;

default: 	 nextstate = S0_FETCH;

endcase

end

// Output Logic

always_comb begin

// When outputs are don’t care, set them to 0

Branch    = 1'b0;

PCUpdate  = 1'b0;

RegWrite  = 1'b0;

MemWrite  = 1'b0;

IRWrite   = 1'b0;

ResultSrc = 2'b00;

ALUSrcA   = 2'b00;

ALUSrcB   = 2'b00;

AdrSrc    = 1'b0;

ALUOp     = 2'b00;

case (state)

S0_FETCH: begin

AdrSrc    = 1'b0;

IRWrite   = 1'b1;

ALUSrcA   = 2'b00;

ALUSrcB   = 2'b10;

ALUOp     = 2'b00;

ResultSrc = 2'b10;

PCUpdate  = 1'b1;

end

S1_DECODE: begin

ALUSrcA   = 2'b01;

ALUSrcB   = 2'b01;

ALUOp     = 2'b00;

end

S2_MEMADR: begin

ALUSrcA   = 2'b10;

ALUSrcB   = 2'b01;

ALUOp     = 2'b00;

end

S3_MEMREAD: begin

ResultSrc = 2'b00;

AdrSrc    = 1'b1;

end

S4_MEMWB: begin

ResultSrc = 2'b01;

RegWrite  = 1'b1;

end

S5_MEMWRITE: begin

ResultSrc = 2'b00;

AdrSrc    = 1'b1;

MemWrite  = 1'b1;

end

S6_EXECUTER: begin

ALUSrcA   = 2'b10;

ALUSrcB   = 2'b00;

ALUOp     = 2'b10;

end

S7_ALUWB: begin

ResultSrc = 2'b00;

RegWrite  = 1'b1;

end

S8_EXECUTEI: begin

ALUSrcA   = 2'b10;

ALUSrcB   = 2'b01;

ALUOp     = 2'b10;

end

S9_JAL: begin

ALUSrcA   = 2'b01;

ALUSrcB   = 2'b10;

ALUOp     = 2'b00;

ResultSrc = 2'b00;

PCUpdate  = 1'b1;

end

S10_BEQ: begin

ALUSrcA   = 2'b10;

ALUSrcB   = 2'b00;

ALUOp     = 2'b01;

ResultSrc = 2'b00;

Branch    = 1'b1;

end

endcase

end

endmodule

// Output Logic

always_comb begin

// When outputs are don’t care, set them to 0

Branch    = 1'b0;

PCUpdate  = 1'b0;

RegWrite  = 1'b0;

MemWrite  = 1'b0;

IRWrite   = 1'b0;

ResultSrc = 2'b00;

ALUSrcA   = 2'b00;

ALUSrcB   = 2'b00;

AdrSrc    = 1'b0;

ALUOp     = 2'b00;

case (state)

S0_FETCH: begin

AdrSrc    = 1'b0;

IRWrite   = 1'b1;

ALUSrcA   = 2'b00;

ALUSrcB   = 2'b10;

ALUOp     = 2'b00;

ResultSrc = 2'b10;

PCUpdate  = 1'b1;

end

S1_DECODE: begin

ALUSrcA   = 2'b01;

ALUSrcB   = 2'b01;

ALUOp     = 2'b00;

end

S2_MEMADR: begin

ALUSrcA   = 2'b10;

ALUSrcB   = 2'b01;

ALUOp     = 2'b00;

end

S3_MEMREAD: begin

ResultSrc = 2'b00;

AdrSrc    = 1'b1;

end

S4_MEMWB: begin

ResultSrc = 2'b01;

RegWrite  = 1'b1;

end

S5_MEMWRITE: begin

ResultSrc = 2'b00;

AdrSrc    = 1'b1;

MemWrite  = 1'b1;

end

S6_EXECUTER: begin

ALUSrcA   = 2'b10;

ALUSrcB   = 2'b00;

ALUOp     = 2'b10;

end

S7_ALUWB: begin

ResultSrc = 2'b00;

RegWrite  = 1'b1;

end

S8_EXECUTEI: begin

ALUSrcA   = 2'b10;

ALUSrcB   = 2'b01;

ALUOp     = 2'b10;

end

S9_JAL: begin

ALUSrcA   = 2'b01;

ALUSrcB   = 2'b10;

ALUOp     = 2'b00;

ResultSrc = 2'b00;

PCUpdate  = 1'b1;

end

S10_BEQ: begin

ALUSrcA   = 2'b10;

ALUSrcB   = 2'b00;

ALUOp     = 2'b01;

ResultSrc = 2'b00;

Branch    = 1'b1;

end

endcase

end

endmodule

aludec.sv 	instrdec.sv	and	controller_testbench.sv 	were given to us by default there were no changes done to them but I have changed the test vectors 		controller.tv

- The controller.tv file was using a RISC-V standard different that the one in the Harris & Harris book and the homework file. I updated the expected ALUControl bits to match the Harris & Harris’s description. The updated file can be found in the GitHub repository.

Example:

| //op[6:0]_funct3[2:0]_funct7b5_Zero_ImmSrc[1:0]_ALUSrcA[1:0]_ALUSrcB[1:0]_ResultSrc[1:0]_AdrSrc_ALUControl[2:0]_IRWrite_PCWrite_RegWrite_MemWrite // add |  |
| --- | --- |
| old | updated |
| 0110011_000_0_0__XX_00_10_10_0_010_1_1_0_0 // Fetch 0110011_000_0_0__XX_01_01_00_0_010_0_0_0_0 // Decode 0110011_000_0_0__XX_10_00_00_0_010_0_0_0_0 // ExecuteR 0110011_000_0_0__XX_00_00_00_0_010_0_0_1_0 // ALUWB | 0110011_000_0_0__XX_00_10_10_0_000_1_1_0_0 // Fetch 0110011_000_0_0__XX_01_01_00_0_000_0_0_0_0 // Decode 0110011_000_0_0__XX_10_00_00_0_000_0_0_0_0 // ExecuteR 0110011_000_0_0__XX_00_00_00_0_000_0_0_1_0 // ALUWB |

As you can see from the error counter there is no errors

https://github.com/Hakan-26/-Multicycle-Processor-Controller